# Spider2 PCIe Driver

# Introduction

This document describes the spider2 PCIe driver.  A single driver is used to communicate with the FPGA over the PCIe bus, as the FPGA appears as a single PCI function.  The spider2 PCIe driver is split into a number of logical sections that reflect the different capabilities and features of the spider2 FPGA.  Further details of the dsp* interfaces can be found in the spider2_pcie_if git repo.

# Driver “dspeng” for Engine Communication

The driver dspeng shall be responsible for communication between the Engine and the FPGA.  All transfers shall be by block reads or writes using the standard read() and write() system calls.  The communication model  is shown in Figure 1.

## Requirements

- New interface to engine (not a socket interface)

- Blocking read() and write() (not polling, though polling and non-blocking operations are allowed)

- Engine to DSP command communication (request)

- Engine to DSP block transfer (1K*32bit)

- DSP to Engine command communication (response)

- Not real-time

## Interface to User-Space

- Character device with blocking read() and write() functions

- Additional settings can be passed or fetched via ioctl()

- All values will be “host endian”

## Interface to FPGA

- PCIe BUS

- FPGA Registers mapped into kernel space (ioread32 and iowrite32)

- Read from “Desc. TX” and “Data TX” via DMA transfer (initiated by FPGA)

- Write to “Desc. RX” and “Data RX” via DMA transfer (initiated by FPGA)

## Engine write()

- Engine starts a write of at most 1K*32Bits (command or block)

  - The data is uint32’s in host endian format

  - Most values for the dsp will only be 24-bit’s

  - A “Host Vector” shall be indicated by setting the MSB of the last word.

The driver copies the data into “Data TX”

- “Desc. TX” is updated: set address and length

- Increment “TX_HEAD” on the FPGA

- FPGA starts to read the “Desc. TX” and copies the content of “Data TX” into “TX FIFO”

## Engine read()

- DSP copies data into the “RX FIFO” and raises an interrupt for the FPGA

- FPGA starts to copy the data into the “Data RX”

- FPGA updates the “Desc. RX”

- FPGA raises an MSI interrupt: Driver catches this interrupt isr()

Driver reads the ISR register of the FPGA to clear interrupt and determine the source

- Driver reads “Desc. RX” and copies the data from “Data RX” to the User-Space

  - The data is uint32’s in host endian order.

Figure 1

# Driver “dspsam” for Sample Value Communication

The driver dspsam is responsible for the communication between the “Sample Value” (SV) application and the FPGA.  The SV application is running as a real-time application.  Therefore we need a fast interface for the Sampled Values (Figure 2).  The communication is unidirectional, no Sampled Values are written to the FPGA.

## User Interface

- Character interface /dev/dspsam for Sample Values

- SV application waits with read() for new data from the FPGA

- SV application falls out of the blocking read() call when new data is available.

Although only one block of data should be available at each read, it is best if the user space application tries to read more than this so that if user space does get “out of step” with the hardware it can catch up quickly.  Of course, if multiple blocks are being transferred per interrupt then multiple blocks will be available to the read.

## FPGA Interface

- Configure a “large” ring buffer in host memory

- New SV data in FPGA copied immediately to host memory ring buffer

- A Counter / Pointer into the ring buffer is updated in host memory

- FPGA generates interrupt

- Driver catches interrupt and signals, in the ISR, that new data is available

We can configure how many blocks are transferred per interrupt.  A block is 32 uint32’s.  1, 2 or 4 blocks will get transferred into the ring buffer at each interrupt.  The counter will be increased by 1, 2 or 4.

## Operation

- The driver allocates a buffer and points the SV_L2H_BUF0_ADDR register to it

- The driver allocates a uint32 counter and points the SV_LOCAL_PKT_CNT_ADDR register to it.

- The FPGA DMA’s data into the hosts memory

- The FPGA increments the counter in host memory

- The driver, in response to a read() from user space, copies the data from the buffer to user space.

- The driver then increments its own local pointer into the buffer.

Figure 2

# Driver “dspgoo” for GOOSE Communication

The driver dspgoo is responsible for communication between the GOOSE application and the FPGA. The GOOSE application is running as a real-time application. Therefore we need a fast interface for the GOOSE data (Figure 3).

###### Driver Interface for GOOSE IN and OUT

- Character interface /dev/dspgoo for GOOSE IN and OUT

- All user-space interactions will be performed with ioctl’s.

- Goose In: A set of values are written to the DSP.

- Goose Out: A set of values is written from the DSP.  This call is blocking, and a timeout can be given in microseconds.

- Abort: Can be called to cancel any pending operations.

All values are “host endian”.

###### FPGA Interface

The Goose OUT interface is identical to the Sampled Value interface but with a smaller block size.  For the dspgoo interface the block size is set to 4 uint32’s so that when the interface is configured for 4 blocks per interrupt we transfer 16 uint32’s.

The Goose IN interface consists of a pair of pointers into host memory.  When the first pointer is set the FPGA initiates a DMA of the data from this pointer to the DSP.  When the DMA has completed, the FPGA writes a zero value to the second pointer.  The driver uses the writing of the zero to the second pointer as an acknowledgement that the transfer has completed.

###### GOOSE IN path

- Construct a structure with the values for the DSP and call the ioctl(GOOSE_IN).

- Values copied from user-space to the driver / PCIe bus.

- The driver blocks until the DSP has consumed the last values written, this is detected by the flag being cleared.

- The driver sets the flag & a pointer to indicate that new values are ready.

###### GOOSE OUT path

- Construct a structure with the required timeout and call the ioctl(GOOSE_OUT).

- Values copied from user-space to the driver.

- The driver waits for an interrupt from the device.

- The driver copies the values from PCIe to user-space.

Figure 3

# Auxiliary Communication

There are a number of functions of the FPGA that are not covered by the three mechanisms above, they are described here:

## LED

One of the LEDs is connected to the FPGA.  This is exposed as the standard Linux LED interface as “LED-B”.  Blinking is supported by this interface, and is performed by the FPGA in hardware.

## Button

The Associate button is connected to the FPGA.  This can be read by a pair of files in the sys file system “/sys/spider2/assoc” and /sys/spider2/assoc_changed”.  The assoc file shows the actual state of the button.  The assoc_changed file is non-zero if the state of the button has changed since the last “clear” of the assoc_changed file.  The assoc_changed file is cleared by writing a value to it.

## IRIGB Time

The IRGB Time can be read by reading the /sys/spider2/irigb file.  The value returned is the 64-bit value returned from the DEC_CUR_TIME and DEC_DUR_DATE registers in the FPGA.

## PoE Interrupt

The PoE interrupt line can be read with the /sys/spider2/poeint file.  A user space program can wait for a change to this file with a poll or select command.  Care is needed here as the file is always readable, the poll call must detect POLLPRI (there is an example called test_poeint in the spider2_pcie git repo).

## HW Status

A few bits of hardware status can be read from /sys/spider2/hw_status.  The bits returned are the least significant four bits of the ASSOC_BUTTON_STATE register (refer to the hardware manual for info in this register).

## Debug

There are a couple of debug interfaces exposed.

The /sys/spider2/debug file shows some debug info as text.  Please refer to the driver source for more info.  This file is subject to change, and might be removed one day.

The /sys/spider2/poke and /sys/spider2/poke_addr files give direct access to the FPGA PCIe memory space.  The address within the FPGA to be accessed is written to the poke_addr file.  A 32-bit value can then be read or written to the FPGA address by reading and writing the poke file.

Obviously, this feature is a minor security hole, and can crash the FPGA.